# Air Quality Analysis Using AirPurple Sensor Data Report

Prepared by: Bhuwanraj Singh Parihar

CIVE 202 – Civil Engineering Analysis II

Civil and Environmental Engineering Department

January 31, 2026

# Table of Contents

1.Introduction................................................................................................................................ 3

2. Methods..................................................................................................................................... 3

3. Results and Discussion.............................................................................................................. 4

3.1 Locations with Highest Pollution ........................................................................................ 4

3.2 Maximum Pollution Events ................................................................................................. 4

3.3 Temperature Influence on Pollution .................................................................................... 4

3.4 Humidity Influence on Pollution ......................................................................................... 4

3.5 Health Risk Assessment ...................................................................................................... 4

3.6 Elevation Effects ................................................................................................................. 5

4. Conclusion ................................................................................................................................ 5

5. References ................................................................................................................................. 5

1. Introduction

The purpose of this project was to analyze air quality data collected from AirPurple monitoring sensors across Nebraska. The client requested an evaluation of pollution patterns, environmental influences, and potential public health risks associated with air quality conditions. Specifically, the project focused on identifying locations with the highest pollution levels, determining when extreme pollution events occurred, evaluating how weather conditions influence air quality, and assessing whether measured pollutant levels reached thresholds associated with health concerns. An additional goal was to explore whether sensor elevation affected measured pollution values. This analysis further supports the client’s objective of understanding environmental conditions that may influence public health outcomes across Nebraska.

2. Methods

Air quality data were provided in CSV format and analyzed using Python and the pandas library. The dataset included daily measurements of volatile organic compounds (VOC), particulate matter smaller than 2.5 micrometers (PM2.5), particulate matter smaller than 10 micrometers (PM10), temperature, humidity, and sensor metadata such as location and elevation.

The dataset was first loaded into a pandas dataframe and to verify column names and data structure. New variables were created to allow grouped comparisons. Temperature values were grouped into four categories (Cold, Cool, Mild, Warm/Hot) based on defined ranges. Humidity values were grouped into three categories (Low, Moderate, High). Sensor elevation values were also grouped into three categories (Low, Medium, High) to evaluate geographic influences.

To identify locations with consistently elevated pollution, the data were grouped and summarised statistically with mean and median for VOC, PM2.5, and PM10. Maximum pollution events were identified using indexing methods to determine the date and location of the highest recorded values for each pollutant.

To evaluate environmental influences, average PM2.5 concentrations were calculated within each temperature and humidity category. PM2.5 values were also classified into EPA Air Quality Index (AQI) categories to assess potential health risks. The number of days in each AQI category was counted to determine how frequently unhealthy air quality levels occurred. Finally, pollutant averages were compared across elevation categories to evaluate whether elevation influenced air quality measurements. All analysis steps were performed using Python, and the code is documented in the project notebook.

3. Results and Discussion

3.1 Locations with Highest Pollution

Summary statistics showed that certain monitoring sites consistently exhibited higher mean and median pollutant concentrations. These locations represent areas where air quality may be more frequently impacted by local emission sources, regional transport, or atmospheric conditions that limit pollutant dispersion.

3.2 Maximum Pollution Events

The highest VOC concentration was recorded on June 24, 2024 at the Ogallala monitoring site. VOC levels can increase during periods of elevated temperatures, vehicle emissions, or industrial activity, suggesting a potential localized emission or atmospheric reaction event.

The maximum PM2.5 and PM10 concentrations both occurred on February 18, 2025 at the Richardson County Courthouse monitoring site. Simultaneous spikes in fine and coarse particulate matter often indicate a regional pollution event such as wildfire smoke transport, wind-driven dust, fireworks, or atmospheric conditions that trapped pollutants near the ground.

3.3 Temperature Influence on Pollution

Average PM2.5 concentrations were highest during cold temperatures and decreased as temperatures increased. This pattern is consistent with winter atmospheric conditions. These results suggest that colder seasons may pose higher risks for elevated particulate matter exposure.

3.4 Humidity Influence on Pollution

PM2.5 concentrations were significantly higher during high humidity conditions compared to moderate or low humidity levels. High humidity can enhance particle growth through moisture absorption and is often associated with stagnant air masses that limit pollutant dispersion. These findings indicate that humid weather conditions may contribute to elevated particulate matter levels.

3.5 Health Risk Assessment (AQI Categories)

Most days in the dataset were classified as Good or Moderate air quality. However, several periods of elevated concern were identified. A total of 119 days were classified as 'Unhealthy for Sensitive Groups,' 10 days as 'Unhealthy,' and 392 days reached 'Very Unhealthy or Hazardous' levels. These high-risk days could have posed health concerns for sensitive populations, including children, older adults, and individuals with respiratory conditions.

3.6 Elevation Effects

PM2.5 and PM10 concentrations were highest at medium-elevation sites and lowest at high-elevation locations, suggesting that mid-elevation areas may experience pollutant accumulation due to local geography or population distribution. In contrast, VOC concentrations were highest at high-elevation sites, indicating that gaseous pollutants may be influenced more by regional atmospheric transport than by local accumulation. This suggests that elevation may influence pollutant behavior differently depending on pollutant type.

4. Conclusion

This analysis identified key air quality patterns across Nebraska, including locations with elevated pollution levels, periods of extreme pollution events, and environmental conditions associated with higher pollutant concentrations. Colder temperatures and high humidity were associated with increased PM2.5 levels, and several days reached AQI categories that could pose health risks. Elevation was found to influence particulate and gaseous pollutants differently. These findings provide the client with valuable insight into when and where air quality may present greater risks to public health.

5. References

U.S. Environmental Protection Agency (EPA). Air Quality Index (AQI) Basics. https://www.airnow.gov/aqi/aqi-basics/

PurpleAir. PurpleAir Data and Sensor Information. https://community.purpleair.com/

AirNow. Air Quality Data Resources. https://www.airnow.gov/