# Scope of Work

Prepared by: Bhuwanraj Singh Parihar

Course: CIVE 202 – Civil Engineering Analysis II

Submitted to: Civil and Environmental Engineering Department

Date Submitted: January 31, 2026

## Project Goals

The goal of this project is to conduct a detailed statistical analysis of air quality data collected from AirPurple monitoring sensors across Nebraska for the UNMC Water, Climate, and Health Working Group. The client has requested the locations with elevated pollution levels, and when it occurred, evaluation of how environmental conditions such as temperature and humidity influence pollutant density, and assessment of whether measured air quality levels posed potential health risks. An additional objective is to explore whether sensor elevation affects pollutant measurements.

## Project Tasks

### Task 1: Data Preparation and Organization

The air quality dataset will be imported into Python using the pandas library. Column names and other data will be reviewed to ensure compatibility with analysis tools. Temperature, humidity, and elevation values will be categorized into defined ranges to support grouped analysis through python.

### Task 2: Location-Based Pollution Analysis

The dataset will be grouped to calculate summary statistics including mean and median values for VOC, PM2.5, and PM10. These results will be used to identify locations with consistently elevated pollution levels. Grouping and aggregation functions in pandas will be used for this analysis.

### Task 3: Identification of Maximum Pollution Events

Maximum recorded values for VOC, PM2.5, and PM10 will be identified using indexing methods to determine the date and monitoring site of extreme pollution events. These events will be evaluated to understand potential environmental causes.

### Task 4: Environmental Influence Analysis

Pollution levels will be compared across temperature and humidity categories to determine how weather conditions influence air quality. Average PM2.5 concentrations will be calculated within each category.

### Task 5: Health Risk Classification

PM2.5 concentrations will be classified according to EPA Air Quality Index (AQI) thresholds to evaluate potential health risks. The frequency of days in each AQI category will be calculated to assess how often unhealthy air quality conditions occurred.

### Task 6: Elevation Impact Assessment

Pollutant concentrations will be compared across sensor elevation categories to determine whether elevation influences air quality measurements. Mean pollutant values will be calculated for each elevation group.

## Project Deliverables

| Deliverable | Files Included | Due to Client |
| --- | --- | --- |
| Raw Data File | AirQuality_Daily_StudentVersion.csv | January 31, 2026 |
| Python Code Notebook | CIVE202_Spring2026_Bhuwanraj_Project1_PythonCode.ipynb | January 31, 2026 |
| Annotated Code Document | CIVE202_Spring2026_Bhuwanraj_Project1_ACD.docx | January 31, 2026 |
| Scope of Work | CIVE202_Spring2026_Bhuwanraj_Project1_SOW.docx | January 31, 2026 |
| Final Written Report | CIVE202_Spring2026_Bhuwanraj_Project1_Report.docx | January 31, 2026 |
| GitHub Repository | GitHub Link to Project | January 31, 2026 |